IP Address Handout

In the Internet, each computer must have a unique IP address for addressing. There are two versions of IP addresses, IPv4 and IPv6; IPv4 refers to an IP address with four bytes, and IPv6 refers to an IP address with 16 bytes. The IP addresses that are still widely used are still mainly IPv4, so this article introduces some concepts of IP addresses based on IPv4.

In IPv4, the address of each computer has 4 bytes, such as 11000000.10101000.00000000.00000001. In order to be easy to remember, we usually express the IP v4 address in decimal, so the above IP address can be expressed as 192.168.0.1. Take the following table as an example, we can see the comparison table between decimal and binary.

Table 1. IPv4 address binary and decimal notation

| binary | decimal |
| --- | --- |
| 10001100.10001010.11000000.01100011 | 140.138.192.99 |
| 11000100.00101010.10100000.00100010 | 196.42.160.34 |
| 10001100.10100010.01110000.10100001 | 140.162.112.161 |

1. Network identification code

Since many countries and organizations in the world need to use a large number of computers, how to assign IP addresses to each country and organization has become a very important task. Now all continents and countries in the world have dedicated agencies to manage the allocation of IP addresses. In order to allocate IP addresses in packages to facilitate operations, usually an organization or a school will be allocated an IP address range and use it as a network identifier. For example, the IP address range assigned to Yuan Ze University is 140.138.0.0 ~ 140.138.255.255. In this address range, the first two bytes remain unchanged, both are 140.138, and the last two bytes allow Yuan Ze University to be flexible. Therefore, 140.138 becomes the network identification code of Yuan Ze University. Table 2 shows the network identification codes assigned by other universities in Taiwan.

Note that for simplicity, we refer to 140.138 as the network identification code. However, a network identification code is always 4 bytes long, so the actual network ID is 140.138.0.0. In other words, the network identification code consists of the network portion of the IP address, with all remaining host bits set to zero.

Table 2. Network identification codes of various universities

National Taiwan University: 140.118 National Chen Chi University: 140.119 National Chung-Shin University:140.120 National Taiwan University of Science and Technology: 140.124

Brainstorming time:

- How many IP addresses can this network have if the network identification code uses the first 24 bits of the IP address?

Answer:

Network identification code uses 24 bits. These bits must be consistent and cannot be changed. Therefore, there are only 8 bits left in the computer address, and each bit can be any value 0 or 1. So we can have 256 different IP addresses.

According to the above, a four-byte IP address in IPv4 can be divided into two parts, as shown in Figure 1, the former part is the network identifier, and the latter part is called the host identifier, or the computer identifier. For example, the first two bytes of the IP address 140.138.192.99: 140.138 are the network identifier, and the last two bytes 192.99 are the computer identifier.

Nework id       Host id

Figure 1. The structure of an IPv4 address

In the IP address, the number of bits used by the network ID and the computer ID is not a constant value, partly because each organization is different in size and requires a different range of IP numbers. Thus, the length of the network ID naturally varies. For example, Yuan Ze University is large in scale, so its address range has 65536 IP addresses, but for smaller schools such as Taoyuan Elementary School, its address range only needs 1024 IP addresses. Then its IP address range can be 145.220.96.0~145.220.99.255. In other words, we leave the last 10 bits of the IP address as the computer identification code, and the remaining first 22 bits as the network identification code.

Table 3 shows the computer number range of Taoyuan Elementary School in binary. We can clearly see that the first 22 bits of 145.220.96.0 and 145.220.99.255 are exactly the same. That is to say, when Taoyuan Elementary School numbers a computer, its IP address cannot change the first 22 digits of the network identification code, but the last 10 digits can be used by Taoyuan Elementary School. We can use binary 1 0001100.11011100.011000xx.xxxxxxxx to represent the labels of all computers in Taoyuan Elementary School, where x means that it can be 0 or 1. For example, 196.220.97.142 in the table below corresponds to one of the above-mentioned computer identification numbers, because its first 22 bits are the same as the identification codes of all other computers.

Table 3. Network marking codes and computer marking codes

| binary | decimal |
| --- | --- |
| 10001100.11011100.01100000.00000000 | 196.220.96.0 |
| 10001100.11011100.01100011.11111111 | 196.220.99.255 |
| 10001100.11011100.01100001.10001110 | 196.220.97.142 |

In computer identification, IPv4 has a rule: every bit in the computer identification code is 0 or all 1 cannot be used as the IP address of a certain computer, because they are used as network identification and broadcast address respectively. As shown in Table 3, 196.220.96.0 must be reserved as the network identification of Taoyuan Elementary School, and 196.220.99.255 must be reserved as the broadcast address.

Brainstorming time:

- If the network identification code uses the first 26 bits of the IP address, how many computers can this network address at most?

Answer:

network identification code uses 26 bits, so there are only 6 bits left for the computer identification code, so we can have 64 different IP addresses. Deducting the use of two IP values for network identification and broadcast addresses, we are left with a maximum of 62 IP addresses as computer addresses.

Brainstorming time:

- Yongfeng High School needs to apply for an IP address. According to the preliminary plan, a total of 500 computers need to be used.

  - If the network identification code assigned to it is 196.220.96.0, then what is its IP address range likely to be?

  - How many digits will there be in the network identification code of Yongfeng High School?

  - Which one of the IP address 196.220.97.22 and the IP address 196.220.98.55 is not the computer address of Yongfeng High School?

Answer:

- 500 computers need more than 500 IP addresses. In binary terms, it requires at least 9-bit computer identification codes, that is, 512 IP addresses, which is enough to give each computer a unique address. In addition to considering the limitations of network identification and broadcast addresses, the 9-bit computer identification code can have up to 510 IP addresses available for use. Therefore, the reasonable IP address range is 196.220.96.0~196.220.97.255.

- According to A, we can see that the first 23 bits of the IP address of Yongfeng High School must remain unchanged. Therefore, its network identification code is 23 bits, and its network identification code is 196.220.96.0.

- If we convert these two IP addresses into binary, we can see that the first 23 bits of 196.220.98.55 are different from the network identification code of Yongfeng High School, so it does not belong to Yongfeng High School.

1-1 Brainstorming time:

- Yongfeng Elementary School needs to apply for an IP address. According to the preliminary plan, a total of 220 computers need to be used.

  - If the network identification code assigned to it is 196.247.99.0, then what is its IP address range likely to be?

2. Subnet design (subneting)

When an organization is configured with an IP address range, it needs to further configure IP addresses within the organization to meet the needs of each unit in the organization. Taking Yuan Ze University as an example, Yuan Ze has many colleges, departments, and divisions. Each unit has a computer requirement, and also needs IP addresses for numbering. Therefore, under the network identification code 140.138, the Information Service Office needs to assign IP addresses to each department and division. This further division of a network into several subnets is called subnetting. For example, the Information Services Department assigns 512 IP addresses to each of the Department of Engineering, Department of Assets Management, Department of Information Communication, etc. So externally, Yuan ze's network identification code is 140.138, but Yuan ze is divided into sub-networks inside, which is the sub-network design. The subnet identification code needs to borrow several bits in the computer identification code as the subnetwork division. Figure 2 is an IPv4 architecture with subnets.

Network ID    Subnet ID      Host ID

Figure 2. The structure of an IPv4 address with subnets

Therefore, the network identification code and the subnet identification code can accurately and uniquely identify a subnet. The number of computers that can be numbered in each subnet is determined by the number of bits in the remaining computer numbers.

Subnet identification code is very important, and the path for transferring IP datagram inside Yuan ze must be determined by it. If IP datagrams are sent to Yuan ze from a network outside Yuan ze, they can use the network identification code 140.138 to select the transmission path. Once these IP datagrams enter Yuan ze campus, each router will further check the destination in each IP datagram. The router, using the length of the router's subnet identification code, extracts the subnet identification code of the destination IP address, and determines its transmission path.

Brainstorming time:

- The network and subnet identification code of the Department of Human Resources and Engineering is 140.138.144, and its computer identification code must have 512 IP addresses.

  - What is its IP address range?

  - What is its subnet ID?

  - How many bits should the subnet identifier borrow from the computer identifier?

answer:

- In binary terms, we need a 9-bit computer identification code , that is, there can be 5 to 12 IP addresses, so the reasonable IP address range is 140.138.144.0~140.138.145.255, that is, 10001100.10001010.1001000 0.00000000 ~ 10001100.1111010101.1001.

- identification code of the subnet is 144, that is, the value when each bit of the computer identification code is 0.

- We borrow a total of 7 bits from the computer identifier for the subnet identifier.

2-1 Brainstorming time:

- Continuing from the previous question, the network and subnet identification code of the Department of Engineering and Engineering is 140.138.144. Now we need to allocate 512 IP addresses to the four areas of multimedia computer classroom, IC design college computer classroom, professor's laboratory, and teaching staff Network, each LAN has 128 IP addresses.

  - What are the subnet identifiers for each LAN?

  - How many bits should the subnet identifier borrow from the computer identifier?

3. Masking

When assigning IP addresses, different organizations, companies or schools assign different ranges of IP addresses, so the lengths of their network identification codes vary. In the router, every time a datagram is received, it must determine the forwarding direction according to the network identification code of its destination IP address. Therefore, it is very important to quickly obtain the network identification code of each IP address. For this, the method of net masking becomes a great solution.

Network masking (Masking) uses a logic value of 1 to represent whether a certain bit is a part of the network identification code and the subnet identification code. For example, the first 23 bits in 140.138.145.224 are the network identification code and the subnet identification code, so we express it as 11111111.11111111.11111110.00000000/23. In decimal, it is 255.255.254.0/23. To obtain the network identification code and subnet identification code of a certain IP address, we only need to make a logical AND of the IP address and the network mask. In the following table, we show how to obtain the network identifier and subnet identifier of 140.138.145.224 in binary form. We can clearly see that the network mask can retain the part of the IP address that belongs to the network and subnet identification code, and all the computer identification code parts will be reset to zero.

Table 4. Example of Obtaining Network and Subnet IDs Using Net Masks

| IP address | 10001100 | 10001010. | 10010001 | 11100000 |
| --- | --- | --- | --- | --- |
| Netmask | 11111111 | 11111111 | 11111110 | 00000000 |
| Netmark | 10001100 | 10001010 | 10010000 | 00000000 |

Brainstorming time:

- The IP address of a computer is 140.138.147.155, and its network mask is 255.255.255.192/26. What is its network and subnet identification code?

Answer: 140.138.147.128

| IP address | 10001100 | 10001010. | 10010011 | 10011011 |
| --- | --- | --- | --- | --- |
| Netmask | 11111111 | 11111111 | 11111111 | 11000000 |
| Netmark | 10001100 | 10001010 | 10010011 | 10000000 |
| decimal representation | 140 | 138 | 147 | 128 |

3-1 Brainstorming time:

- The IP address of a computer is 140.138.147.155, and its network mask is 255.255.255.0/24. What is its network and subnet identification code?

- There are three computers whose IP addresses are computer A: 140.138.147.155, computer B: 140.138.147.193, and computer C: 140.138.147.97. If the network mask is 255.255.255.128/25, which computers belong to the same subnet? What are their network and subnet identification codes?

4. Network mask setting

When building a network, we need to set the network mask in each device as the basis for path selection. In the Internet, the IP layer is responsible for selecting the path, so whether it is a terminal computer or a router, path selection is required. Therefore, we need to set a network mask in these devices.

Why does the terminal computer need to set up a network mask? The reason is very simple. When the terminal computer A wants to send an IP datagram to another computer, the terminal computer A needs to decide whether to send the IP datagram to its preset service router for forwarding, or to send the IP datagram directly to the destination computer. How does the terminal computer A decide? It only needs to calculate the network mask of A and the destination IP address of the IP datagram, and extract its network identification code and subnet identification code (hereinafter collectively referred to as network identification codes), and then compare it with your own network identification code. If they are the same, it means that they are located in the same subnet, so the terminal computer can directly transmit through the LAN. If they are different, it means that the destination computers are located in different LANs. Therefore, the IP datagram needs to be forwarded through the service router. Take the following figure as an example, computers A and B are located on the same LAN, and router X is their service router. Computers C and D are located on the same LAN, and router Y is their service router. Therefore, if A wants to send datagrams to B, since their network identification codes are the same, A directly sends the datagrams to B through the switch (Ethernet Switch). If A wants to send Datagram to C, A will find that C's network identification code is different from it, so A will send Datagram to router X for forwarding.

Computer C

Computer B

Computer D

路由器X

路由器Y

電腦 A

Figure 3. Subnet Mask Routing Example

If you are on your Windows computer, click on the network connection in the console, and select support in the regional connection status, you can get the information in Figure 4. It includes the IP address of the terminal computer, the subnet mask, and the service router (ie, the default gateway). If you select detailed information, you can also see the DNS address and the hardware address (or MAC address) of the terminal computer.

Figure 4. The mask setting and default gateway (or service router) of the terminal computer

On the router, we must also set the netmask for each network connected to the router. The network connected to each port of the router must use a different network identification code, so that the selection of the transmission path can be correct. Since the main function of the router is to connect multiple networks, the router must be connected to more than one network, that is to say, each port of a router belongs to different networks, so the router is also called multi-home equipment. As shown in Figure 5, the Port at the bottom of the router belongs to the Marketing subnet, while the Port at the top belongs to the Engineering subnet. In other words, if the network identification code of the Marketing subnet is 140.138.121.192, the network identification code of the Engineering subnet is 140.138.121.128, and their network masks are both 255.255.255.192/26, the router must set its network mask and its network identification code are used as the basis for path selection.

Since each port of the router belongs to different subnets, their IP addresses must also match the network identification codes of the subnets they belong to. Taking Figure 5 as an example, the IP address of the port at the bottom of the router must have a network identifier of 140.138.121.192, and the IP address of the port at the top must have a network identifier of 140.138.121.128. Therefore, the address of the port at the lower end of the router can be 140.138.121.194; and the port at the upper end can be 140.138.121.135. In fact, if I know the IP address of a router's Port, plus its network mask, we can know the network identification code of the subnet, so we don't need to set the network identification code of each port separately code.

Figure 5. The router is a multi-home device

Brainstorming time:

- A router has three Ports A, B and C. Their IP addresses are 140.138.147.55, 140.138.147.155 and 140.138.147.78 respectively. The network mask is 255.255.255.192/26. What is the NetID for the subnet of each port connected to?

Answer: The network identifier for port A is 140.138.147.0, the network identifier of Port B as 140.138.147.128, and the network identifier of Port C is 140.138.147.64.

Brainstorming time:

- A router has three ports A, B, and C. Their IP addresses are 140.138.147.55, 140.138.147.155, and 140.138.147.78 respectively, and their network mask is 255.255.255.192/26. The IP address of one router is 140.138. 147.181, the network mask is 255.255.255.192/26, what is the IP address of the service router?

Answer: By logical AND operation of the network mask and its IP address, we can obtain its network identification code as 140.138.147.128, which is the same as the network identification code of router Port B. Therefore, the address of its service router should be router Port B's 140.138.147.155.

4-1 Brainstorming time:

- A router has three Ports A, B and C. The network identification codes are 140.138.147.0, 140.138.147.192 and 140.138.147.128 respectively. The network mask is 255.255.255.192/26. Set a suitable IP address for each port.

4-2 Brainstorming time:

- A router has three ports A, B, and C. Their IP addresses are 140.138.147.55, 140.138.147.155, and 140.138.147.78. The network mask is 255.255.255.192/26. The IP address of one computer is 140.138.147.105. What is the IP address of its service router?

4-3 Brainstorming time:

- A router has three Ports A, B, and C. Their IP addresses are 140.138.147.55, 140.138.147.155, and 140.138.147.78. The network mask is 255.255.255.192/26. What is a reasonable host IP address range value for the subnet connected to port C?

4-4 Brainstorming time:

- We have to configure a network that supports two PC labs and an office. Current design is as follows:

Subnet A: 60 PCs

Subnet B: 80 PCs

Subnet C: 20 PCs

The IP addresses for the three subnets must be in the range of 140.138.143.0~140.138.143.255

- Define the mask for each subnet,

- Specify the netID for each subnet,

- Give the range of host addresses in each subnet.

- The serving gateway of each subnet